NEWS RELEASE

Contact: Clyne Media, Inc.
Tel: (615) 662-1616

Fax: (615) 662-1636

FOR IMMEDIATE RELEASE

AUDIO-TECHNICA DISPLAYS 

AT2021 CARDIOID CONDENSER MICROPHONE
— Designed for home studio and stage applications, AT2021 provides extended frequency response, high maximum SPL and a wide dynamic range, 

all at a budget-conscious price point —

129th AES CONVENTION, SAN FRANCISCO, CA, November 5, 2010 — Audio-Technica (booth 802), a leading innovator in transducer technology for over 45 years, offers the new AT2021 Cardioid Condenser Microphone. Ideal for home studio and live applications, the low-profile AT2021 offers an extended frequency response, high maximum SPL and wide dynamic range. The microphone’s condenser design provides studio-quality performance for vocal and instrument applications, and is outstanding for acoustic guitar, overheads, piano and group vocals. The AT2021 is also available packaged with the acclaimed AT2020 in the AT2041SP Studio Pack. 

The AT2021 captures smooth, natural sonic characteristics of the audio source, and the low-mass element provides excellent transient response. Corrosion-resistant contacts from the microphone’s gold-plated XLRM-type connector, in addition to the mic’s rugged design and construction, ensure consistent, reliable performance and durability for the user. The unit’s cardioid polar pattern rejects pickup of sounds from the sides and rear, improving isolation of the desired sound source.

The AT2021 specifications include:

Element

 

Fixed-charge back plate, permanently polarized condenser

Polar Pattern

 

Cardioid

Frequency Response 

30-20,000 Hz

Open Circuit Sensitivity 

–39 (11.2 mV) re 1V at 1 Pa

Impedance



250 ohms

Maximum Input Sound Level 
145 dB SPL, 1 kHz at 1% T.H.D.

Noise



 
19 dB SPL

Dynamic Range (Typical)
 
126 dB, 1 kHz at Max SPL

Signal-to-Noise Ratio
 
75 dB, 1 kHz at 1 Pa

Phantom Power Requirements 
48V DC, 2 mA typical

Weight


 
83 g (2.9 oz)

Dimensions


 
101.5 mm (4.00") long, 

21.0 mm (0.83") maximum body diameter

Output Connector

 
Integral 3-pin XLRM-type

Accessories Furnished
 
Stand clamp; soft protective pouch

Audio-Technica’s AT2021 Cardioid Condenser Microphone is now available with a U.S. MSRP of $139.00. For more information, please visit www.audio-technica.com.
…ends 269 words

Photo Files: AT_AT2021_Photo1.JPG, AT_AT2021_Photo2.JPG

Click here for AT2021 product profile and downloadable high-res photo(s).
See more Audio-Technica news releases.
Celebrating over 45 years of audio excellence worldwide, Audio-Technica is a leading innovator in transducer technology, renowned for the design and manufacture of microphones, wireless microphones, headphones, mixers, and electronics for the audio industry.

—For more information on the complete range of Audio-Technica products, contact 

Karen Emerson, Audio-Technica U.S., Inc., 1221 Commerce Drive, Stow, OH 44224. 

Tel: (330) 686-2600; Fax: (330) 688-3752; Web: www.audio-technica.com
—For further information regarding product availability and pricing in the United Kingdom, please contact Harvey Roberts (hroberts@audio-technica.co.uk); in Germany, please contact Anja Groth (a.groth@audio-technica.de); in the rest of Europe, please contact Evangéline Pirot (epirot@audio-technica-europe.com).

Audio-Technica is exhibiting at Booth 802 at the 129th AES Convention in San Francisco, CA.
•3•


